Barn Fires: What you can do now!

Barn fires are one of the most common emergencies at equine facilities. A little bit of work and preparation now can help prevent heartache or panic in the future.

Prevention begins by looking around your barn and property:

- Most barn fires are caused by faulty electrical, so farm owners should be checking wiring continuously, be careful with extension cords, and electrical components are free of dust or cobwebs

- Smoking should never be allowed around barns

- Keep combustibles away from ignition sources – blankets in winter near heat sources

- Sweep up excess straw and hay, removing fuel sources

- Clear the exterior perimeter of your barn of weeds, wood piles, debris as a fire break. Ideally, have at least 3 feet clear

- Keep barn aisles and farm yards clean and clutter free

- Buildings should have lightning rods installed

- Keep halters and lead ropes for each horse on their stall to help with a quick evacuation

Common Causes of Barn Fires:

- Cigarettes and matches

- Sparks from welding/machinery/farrier

- Motors

- Heaters

- Electrical Appliances

- Fence Chargers

- Faulty wiring and fixtures

- Batteries

- Chemicals

Plan Ahead: A monthly inspection and maintenance plan will seem intimidating at first, but once it is underway, the time to inspect and maintain will reduce greatly!

- Involve your boarders by assigning them an inspection task each month

- If you are a boarder, approach the owner about a plan or with offers of assistance

- Keep a record of your inspections and maintenance activity. You will find it helpful to see where problems re-occur and will help you lay out an annual maintenance plan and forecast future facility needs

Fire Extinguishers:

- If the fire is small, a fire extinguisher can quickly put it out. However, if the fire is bigger than an extinguisher can handle, wait for the professionals.

- Always read the labels to ensure they are properly rated for the type of building and conditions

- Generally, an ABC rated extinguisher will be appropriate for most barns

- Place extinguishers at every entrance, in every vehicle, and in all the surrounding outbuildings

- Check them periodically to see that they are fully charged, and at the same time, give them a shake to loosen the chemicals inside

Smoke Detectors and Sprinklers:

- Smoke detectors like those used in homes are not recommended, as the environment in a barn will render them useless

- Flame, heat, and thermal detectors are good options, but have a professional install them and link to a central monitoring system

- Sprinkler systems are a proven and effective fire prevention tool, but the cost of a sprinkler system will depend on your location and water source

Have A Plan!

- Have an evacuation plan in place should a fire occur and make sure all boarders, workers, and managers know what to do in case a fire starts in any location on property

- Have emergency numbers posted around property, with the farm address posted on the list in case the person calling doesn’t know it

- Make sure neighbors have your phone number to contact you in case they see something while you are not home

- Plan fire drills with all people involved with your farm so there is no confusion when an emergency occurs. Mix up the time and scenarios

- Invite local first responders to your farm so they can become familiar with your layout. Show them how to access the barns, stalls, and where water sources are located

- Make sure your address is clearly visible from the roadway

Planning ahead today can help prevent a disaster in the future. Taking small steps today will make you prepared for tomorrow.

Please visit

cthorsecouncil.org

for more information